## Videotext Windscreen Repair

The complete windscreen repair kit comes in one box, including the instructions manual and all necessary materials.

After verifying that a crack can be repaired, the area around the crack needs to be cleaned. Make sure that no cleaning agent enters the crack.

Now, attach the mirror to the inside of the windscreen behind the crack. This provides good visibility of the damaged area. Next, loose splinters of glass must be removed and the glass has to be cleaned again. Then select the appropriate type of repair resin, depending on the crack.

The injector is required to inject the repair resin into the crack. Fix the injector bracket to the windscreen using the suction cup and align the injector exactly above the centre of the crack. Turn the injector clockwise until it contacts the glass and then turn it another quarter of a turn. Now turn the tension adjustment screw until it touches the glass.

Pour in a few drops of the selected repair resin, until the centre of the crack is completely covered.

Connect the combined pressure/vacuum pump to the black marked side of the hose. Connect the other side of the hose to the injector.

Pull the knurled slider toward the pressure gauge to switch the pump to the pressure mode. Apply a pressure of approx. 0.7 to 1.4 bar to the injector, until the repair resin has filled the crack.

Now push the knurled slider away from the pressure gauge to switch the pump to the vacuum mode, apply a vacuum of 0.5 to 0.8 bar and hold this vacuum for approximately 10 minutes.

Proceed with the second pressure phase. To do so, apply a pressure of approximately 1.4 to 2.1 bar for 5 to 10 minutes. Then release the pressure.

Remove the injector and the bracket and lay a resin hardening foil onto the crack.

Position the UV-light above the crack using its two brackets with suction cups, and switch on the UV-light for approximately 5 minutes.

Remove the UV-light from the windscreen.

Then, with the aid of a razor blade, take the hardening foil off the repaired area and remove any excess hardened repair resin from the windscreen.

Remove the mirror from the inside and clean the windscreen from both sides with glass cleaner.

## Videotext Exterior Plastic Repair

The exterior plastic repair kit is delivered in one box, including the instructions manual, different adhesives and material for preparation and cleaning of the parts to be repaired.

Let’s have a look at this bumper, and how to repair such a deep scratch that goes through to the other side.

Cut or mill out the front side of the tear in a V shape, and sand down the front area to be repaired.

Then sand down the rear area to be repaired.

Spray the area with plastic cleaning agent and allow the agent to act for 2 minutes. Then wipe with a clean, dry, lint free cloth.

Apply a thin layer of plastic primer and allow to air for 2 minutes.

Cut the self-adhesive reinforcement tape to the required size for the repair.

Stick the adhesive tape directly to the rear side of the damaged area.

Choose the appropriate adhesive, depending on the kind of repair.

To achieve a homogenous 1:1 mixture squeeze out 1 to 2 cm of the adhesive.

If necessary, apply some adhesive to the rear side of the tear and smooth it using a spatula.

Then prepare the front side using a plastic cleaning agent and plastic primer.

Cut the contour foil to the required size.

Apply adhesive from the front through the tear, and smooth it with the aid of the contour foil and a spatula.

After the adhesive has hardened, remove the contour foil and sand the surface.

Then clean the area with silicon cleaner, apply primer and a final paint coat finish.

## Videotext Smart Dent Repair

A small dent that doesn’t damage the paint can easily happen

That is when you can use the Mini Lifter to remove the dent without doing any paintwork.

The dent puller set comes in one box, including the instructions manual and most of the required material and tools.

The plastic feet adapt themselves to body contours.

A dent is not only an impact, there is also a raised area around the circumference, to which the Mini Lifter plastic feet then apply pressure while pulling out the dent.

A dent in a can, can be removed when applying pressure to the edges of the dent. The same principle is also used by the Mini Lifter.

Rubber feet can be attached to the Mini Lifter in order to protect the paint even more.

A scale can be used to measure the distance pulled.

The various adapters are very important. After selecting the appropriate adapter, special hot glue is applied to it and the adapter is positioned exactly in the centre of the dent.

Insert the Mini Lifter onto the adapter and carefully lift out the dent. It helps to rotate the Mini Lifter while pulling, so the feet do not always push against the same area.

With the aid of a neon lamp you can check whether the dent is still there. When the dent is still too deep, the light source bulges out around the circumference of the dent. When the dent is lifted out too far, the light source appears pinched. An excessively lifted dent can be corrected by using the dressing cone.

And this is how it looks when the dent has been completely removed.

Notes: