## On-board Diagnostic System

### General

- The OBD (On-Board Diagnostic) system is integrated in the PCM and checks the overall engine management system for malfunctions. If a malfunction is detected, a corresponding diagnostic trouble code is stored in the fault memory. Then the malfunction indicator light is illuminated in order to alert the driver to the malfunction. A serial interface allows access to the engine-related diagnostic information using M-MDS, providing the basis for an efficient diagnosis and repair.

L3003_02001

| 1 | PCM | 9 | Simulation test function |
| --- | --- | --- | --- |
| 2 | OBD system | 10 | PID monitor function |
| 3 | MIL driver | 11 | Fault detection function |
| 4 | Malfunction indicator light | 12 | Fail-safe function |
| 5 | Fault memory | 13 | Sensors |
| 6 | Serial interface | 14 | Engine management system |
| 7 | Data link connector | 15 | Actuators |
| 8 | M-MDS |  |  |

## Malfunction Indicator Light

- The MIL (Malfunction Indicator Light) is located in the instrument cluster and serves to alert the driver to a malfunction in the engine management system. During normal operation the MIL illuminates when the ignition is on and is extinguished when the engine has started.

- If the MIL stays on after engine start or comes up continuously during driving, then the OBD system has detected a fault causing increased exhaust emissions.

- If the MIL starts to flash after engine start or during driving, then the OBD system has detected severe misfire causing irreversible damage to the TWC.

NOTE:	If the MIL does not illuminate when the ignition is on, or stays on or flashes after engine start but no fault is stored in the PCM memory, this indicates a malfunction in the MIL control circuit.

L3003_02002

## Data Link Connector

- The DLC (Data Link Connector) is the interface between the OBD system and M-MDS, and allows access to the engine-related diagnostic information.

- All EOBD vehicles are equipped with a 16-pin DLC (also termed as DLC-2) in the passenger compartment (near the steering column). The connector and the data communication protocols are standardized, i.e. the engine-related diagnostic information can also be accessed using a generic scan tool.

L3003_02003

- Some Mazda vehicles are additionally equipped with a 17-pin DLC (also termed as 
DLC-1) in the engine compartment (near the left suspension strut). The connector is not standardized, i.e. the engine-related diagnostic information can only be accessed using M-MDS.

- Furthermore, the 17-pin DLC features eight additional pins, which allow to retrieve an engine speed signal or to activate the fuel pump.

L3003_02004

## Diagnostic Trouble Codes

- After a fault has been detected a corresponding DTC (Diagnostic Trouble Code) is stored in the PCM memory and can be read out using M-MDS. A DTC is defined as a 5-digit alphanumerical code (e.g. P0116) containing the information shown below.

### DTC Status

- When reading out CMDTCs (Continuous Memory DTCs) on a vehicle with HS-CAN bus, the M-MDS additionally displays a status information behind each DTC (e.g. P0231-60). A corresponding description for the DTC status can be found on the right-hand side of the M-MDS screen.

L3003_02023

- The DTC status is defined as a two-digit numerical or alphanumerical code. The first digit indicates, whether the fault is present at the time of DTC read-out and whether the MIL is on for this fault. The second digit provides information about the fault symptom (e.g. signal above or below threshold, no signal etc.). The following status information is used:

## Fault Detection Function

- The fault detection function is integrated in the PCM and monitors the sensors, actuators and PCM-internal components for malfunctions. Different monitoring strategies are used depending on the component concerned.

### Monitoring Strategy for Sensors

- The sensors of the engine management system are monitored for electrical faults, range faults and plausibility faults. Therefore, the fault detection function measures the signal voltage of the sensors.

L3003_02006

| 1 | Sensor | 3 | Voltage detection circuit |
| --- | --- | --- | --- |
| 2 | PCM |  |  |

#### Electrical Faults

- In order to detect electrical faults the fault detection function continuously compares the signal voltage measured to the limits for open circuit and short circuit to ground. If the signal voltage exceeds the upper limit (e.g. more than 4.8 V), the fault detection function determines that an open circuit exists. If the signal voltage measured exceeds the lower limit (e.g. less than 0.2 V), then this indicates a short circuit to ground.

L3003_02033

| X | Physical parameter (here: pressure) | Y | Output voltage |
| --- | --- | --- | --- |
| 1 | Open circuit | 4 | Correct signal |
| 2 | Range fault | 5 | Faulty signal |
| 3 | Short circuit to ground |  |  |

- Range Faults

- In order to detect range faults the fault detection function continuously compares the signal voltage measured to the measuring range of the sensor. If the signal voltage exceeds the specified range (e.g. less than 0.5 V or more than 4.5 V), the fault detection function determines that a range fault exists.

#### Plausibility Faults

- In order to detect plausibility faults the fault detection function compares the signal voltage measured to other parameters of the engine management system by means of logical aspects. These parameters are derived from the momentary operating conditions of the engine and define an expected range for the sensor signals.

- If the signal voltage exceeds this range (e.g. the difference between the mass air flow detected by the MAF sensor and the mass air flow derived from the TP sensor signal is too high), the fault detection function determines that a plausibility fault exists.

### Monitoring Strategy for Actuators

- The actuators of the engine management system are monitored for electrical faults and functional faults. Therefore, the fault detection function measures the control voltage or the control current of the actuators.

L3003_02008

| A | Monitoring the control voltage of the actuator | B | Monitoring the control current of the actuator |
| --- | --- | --- | --- |
| 1 | From PCM control relay | 4 | Voltage detection circuit |
| 2 | Actuator | 5 | Current detection circuit |
| 3 | PCM |  |  |

#### Electrical Faults

- In order to detect electrical faults the fault detection function energizes the actuator in question for a short time (e.g. after switching the ignition on or during engine start) and compares the control voltage/control current measured to the limits for faulty control circuit, open circuit and short circuit to ground.

- If the control voltage measured exceeds the upper limit (e.g. more than 8 V) when the actuator is energized, the fault detection function determines that the control circuit is faulty. If the control voltage measured exceeds the lower limit (e.g. less than 8 V) when the actuator is de-energized, then this indicates an open circuit or a short circuit to ground.

- If the control current measured exceeds the lower limit (e.g. less than 1 A) when the actuator is energized, the fault detection function determines that the control circuit is faulty, or that there is an open circuit or a short circuit to ground.

L3003_02009

| X | Time | Y1 | Ignition switch status |
| --- | --- | --- | --- |
| Y2 | Actuator status | 3 | Actuator energized |
| 1 | Ignition ON or engine start | 4 | Actuator de-energized |
| 2 | Ignition OFF |  |  |

#### Functional Faults

- In order to detect functional faults the fault detection function monitors other parameters of the engine management system by means of logical aspects, when the actuator in question is activated. These parameters are derived from the momentary operating conditions of the engine and show the reaction of the system that is influenced by the actuator.

- If the reaction of the system does not correspond to the control signals output to the actuator (e.g. the engine speed detected by the CKP sensor does not increase when the IAC valve is opened), the fault detection function determines that a functional fault exists.

### Monitoring Strategy for PCM

- The components of the PCM (e.g. FEEPROM, RAM) are monitored for functional integrity by the PCM-internal hardware and software. Many of the diagnostic routines for the PCM are performed when the ignition is switched on or when the engine is running. Other routines that require a considerable amount of processor capacity (e.g. diagnostic routines for the FEEPROM) are carried out during the power-latch phase of the PCM. This prevents the diagnostic routines from interfering with other PCM operations.

## Fail-safe Function

- The fail-safe function is integrated in the PCM and ensures the drivability of the vehicle during the occurrence of a fault. Different fail-safe strategies are used for sensors and actuators.

### Fail-safe Strategy for Sensors

- If a fault is detected on a sensor for which fail-safe data is stored in the PCM, the fail-safe function reverts to a constant substitute value for engine control. In order to prevent any further damage the fail-safe function will always use a “safe” substitute value. As a result, the vehicle can be driven without major restrictions.

- If e.g. the ECT sensor is faulty, the PCM uses the engine coolant temperature of the cold engine (e.g. -40 °C) for engine control to ensure that the engine operates properly even at low ambient temperatures. In addition, the cooling fan is activated continuously to prevent the engine from overheating.

NOTE:	If the fail-safe function uses a constant substitute value due to a faulty sensor, this value is also displayed when monitoring the calculated value (e.g. temperature, pressure etc.) of the corresponding PID using M-MDS. In order to prevent mis-diagnosis the voltage value of the PID concerned should be monitored as well. If e.g. an open circuit exists on the ECT sensor, the PID ECT (Temp) displays a value of 
-40 °C (substitute value) but the PID ECT (Volt) indicates a value of 4.6 V (open circuit).

- If a fault is detected on a sensor for which no fail-safe data is stored in the PCM, the fail-safe function initiates a limp home mode. As a result, the vehicle can be driven either with reduced driving comfort or with limited power output (depending on the sensor affected).

### Fail-safe Strategy for Actuators

- If a fault is detected on an actuator, the fail-safe function deactivates the corresponding control circuit and initiates a limp home mode. As a result, the vehicle can be driven either with reduced driving comfort or with limited power output (depending on the actuator affected).

NOTE:	The fail-safe function might also deactivate the control circuit of an actuator, if an input signal required for control of the actuator in question is faulty. In order to rule out a faulty actuator, perform a simulation test using M-MDS. If the actuator can be controlled via M-MDS, check the relevant input signals for malfunctions.

## European On-board Diagnostics

### General

- The European Union is introducing progressively stricter emissions legislation to achieve a long-lasting reduction in air pollution from vehicle emissions. As part of this legislation, all new petrol passenger vehicles requiring type approval from January 1st 2000 must be equipped with the EOBD (European On-Board Diagnostics) to monitor exhaust emissions. In addition, all petrol passenger vehicles first registered from January 1st 2001 must meet the EOBD regulations.

- The EOBD system uses no additional sensors or actuators to measure the pollutants in the exhaust gas. It is integrated in the PCM and generally uses the existing sensors and actuators of the engine management system plus special software. These check the emission-relevant systems and components while the vehicle is traveling, and calculate the exhaust emissions accordingly.

- If any changes that significantly deteriorate exhaust emissions are detected, the MIL is illuminated continuously. This is to alert the driver to the fact that the vehicle is potentially producing more emissions than it should, and the cause should be investigated as soon as reasonably practicable.

- However, if severe misfire that causes irreversible damage to the TWC is detected, the MIL flashes. In this case, the driver should reduce the vehicle speed immediately and have the vehicle checked as soon as possible.

NOTE:	If the MIL continues to flash after the vehicle speed has been reduced, heavy acceleration should be avoided to prevent the TWC from being damaged.

### Type Approval and Testing

- According to the EOBD regulations the operation of the EOBD system must be guaranteed over the entire life of the vehicle. To obtain type approval for the European market, a vehicle must comply with the specified exhaust emission limits for at least 80,000 km (vehicles with Euro 3 emission standard) / 100,000 km (vehicles with Euro 4 emission standard) or five years (whichever comes first).

- Compliance of mass production vehicles with the specified exhaust emission limits will be monitored in future by the authorities. Therefore, vehicles with various mileages will undergo random testing. If these checks reveal that the specified limits are exceeded systematically, the vehicle manufacturer will be held responsible. This may lead to costly recalls or restrictions to the type approval.

- The exhaust emission limits used by the EOBD system to activate the MIL are always slightly higher than the limits of the Euro 3 emission standard. As a result, minimal overshooting of the Euro 3 limits still does not lead to an activation of the MIL.

|  | CO (g/km) | HC (g/km) | NOX (g/km) |
| --- | --- | --- | --- |
| Euro 3 | 2.3 | 0.2 | 0.15 |
| Euro 4 | 1.0 | 0.1 | 0.08 |
| EOBD | 3.2 | 0.4 | 0.6 |

L3003_02010

| X | Mileage | Y | Exhaust emissions |
| --- | --- | --- | --- |
| 1 | Exhaust emission limits of EOBD | 4 | Actual exhaust emissions of the vehicle |
| 2 | Exhaust emission limits of Euro 3 emission standard | 5 | Emission relevant fault |
| 3 | Exhaust emission limits of Euro 4 emission standard | 6 | MIL activated |

### Definitions

- The following definitions are important to understand the operation of the EOBD system.

#### Drive Cycle

- According to the EOBD regulations a drive cycle consists of engine start-up, engine operation where a malfunction would be detected if present, and engine shut off.

NOTE:	A drive cycle starts when the engine exits from the "Engine Start" state and is completed when the ignition is switched off and the power latch phase of the PCM has ended. However, if the ignition is switched back on during the power latch phase the drive cycle is not completed.

#### Warm-up Cycle

- According to the EOBD regulations a warm-up cycle consists of engine start-up and sufficient engine operation, so that the engine coolant temperature has risen at least 22 °C from engine starting and reaches a minimum temperature of 70 °C.

### Monitors

- The so called monitors are diagnostic routines carried out by the PCM in order to detect malfunctions of the emission-relevant systems and components. On Mazda petrol engines continuous and non-continuous monitors are used.

- Continuous monitors operate permanently, i.e. the correct function of a system/ component is checked permanently during a drive cycle.

L3003_02011

| X | Time | Y | Vehicle speed |
| --- | --- | --- | --- |
| 1 | Engine start | 3 | Monitor operative |
| 2 | Engine shut-off | 4 | Monitor inoperative |

- The continuous monitors are activated after engine start and operate until the engine is shut off.

- Non-continuous monitors operate intermittently, i.e. the correct function of a system/ component is checked only once during a drive cycle, when suitable engine operating conditions exist.

L3003_02012

| X | Time | Y | Vehicle speed |
| --- | --- | --- | --- |
| 1 | Vehicle speed when the monitoring conditions are met | 4 | Monitor operative |
| 2 | Engine start | 5 | Monitor inoperative |
| 3 | Engine shut-off |  |  |

- The non-continuous monitor is activated as soon as the operating conditions required for monitoring (also termed as monitoring conditions) exist, and the respective tests are carried out in a certain order. If the required operating conditions are no longer met during the monitoring phase, monitoring is stopped and the data collected so far is frozen. After regaining the operating conditions during the same drive cycle, monitoring is continued. In case the operating conditions are not met again during the same drive cycle, monitoring starts anew during the next drive cycle.

- When monitoring is completed and no fault has occurred, the monitor is deactivated. As a result, a fault occurring in the system/component after the monitor has been deactivated will only be detected during the next drive cycle.

- If a fault is detected during the monitoring phase, the system/component concerned and the monitor are switched off until the next drive cycle starts. In case the fault also affects the operation of other monitors, these are switched off as well. In this way, storage of subsequent faults is prevented.

- The EOBD system of Mazda petrol engines comprises the following monitors:

- Comprehensive component monitor

- Misfire monitor

- Fuel system monitor

- HO2S monitor

- HO2S heater monitor

- TWC monitor

- EGR system monitor (depending on the vehicle type)

#### Comprehensive Component Monitor

- The CCM (Comprehensive Component Monitor) continuously checks the emission-relevant sensors and actuators for electrical faults, range faults, plausibility faults and functional faults. If an emission-relevant component fails, this is recognized by the monitor and a fault is stored in the PCM memory.

- The CCM only monitors those emission-relevant sensors and actuators, which are not checked by another monitor. In addition, the monitor checks components that are used by the EOBD system, but have no influence on the exhaust emissions.

#### Misfire Monitor

- The misfire monitor continuously checks, whether the acceleration of the crankshaft during the working stroke of each cylinder is above a certain limit. The acceleration of the crankshaft depends on the amount of torque produced by each cylinder. A misfiring cylinder reduces the amount of torque produced and hence the acceleration of the crankshaft.

- The misfire monitor compares the crankshaft acceleration during the working stroke of each cylinder (detected via the CKP sensor signal by additionally using the CMP sensor signal) to the average value of the crankshaft accelerations of the other cylinders during two crankshaft revolutions to determine, if any of the cylinders is not producing the expected amount of torque. Crankshaft accelerations that deviate too much from the average value of the other cylinders indicate a misfiring cylinder.

L3003_02030

| X | Time | Y | Output voltage |
| --- | --- | --- | --- |
| 1 | Normal CKP sensor signal | 5 | Working stroke of cylinder no.4 |
| 2 | CKP sensor signal when misfire occurs | 6 | Working stroke of cylinder no.2 |
| 3 | Working stroke of cylinder no.1 | 7 | Time deviation due to misfire at cylinder no.4 |
| 4 | Working stroke of cylinder no.3 |  |  |

- If misfire is detected, the PCM counts the number of misfires over 200 and 1000 crankshaft revolutions, and calculates the total misfire rate and the misfire rate for each cylinder:

Misfire rate = No. of misfires / No. of working strokes

- E.g. a total misfire rate of 25 % means that one cylinder in a 4-cylinder engine misfires completely.

- If the misfire rate over 200 crankshaft revolutions exceeds the limit (typical values range from 40 % at idle to as low as 5 % at high engine speed and load) and the TWC has reached its operating temperature (calculated by the PCM), this is classified as a severe misfire causing irreversible damage to the TWC.

- If the misfire rate over 1000 crankshaft revolutions exceeds the limit (typically 1…3 % regardless of engine speed and load), this is classified as a minor misfire causing increased exhaust emissions only.

- If a cylinder misfires and subsequently the misfire rate over 200 or 1000 crankshaft revolutions exceeds the limit, this is recognized by the monitor and a fault is stored in the PCM memory.

NOTE:	The Tribute F/L features a special strategy to protect the TWC against damage in case of severe misfire. If such a misfire is detected, the PCM cuts off the fuel injection at the cylinder in question. As a result, no unburnt fuel is induced into the TWC and burnt there, preventing the catalytic converter from being damaged.

#### Fuel System Monitor

- The fuel system monitor continuously checks, whether the short-term and long-term fuel trim values of the fuel system are within a certain range. If a fuel system-related component fails and subsequently the correction factors of the short-term and long-term fuel trim exceed the rich or lean limit (typically ± 25 %) for more than a specified duration, this is recognized by the monitor and a fault is stored in the PCM memory.

#### HO2S Monitor

- The HO2S monitor intermittently checks the upstream and downstream HO2S for range faults and plausibility faults. In addition, the monitor checks the upstream HO2S for dynamic faults. Therefore, the PCM uniformly varies the air/fuel ratio around λ = 1, and monitors the response time and period duration of the HO2S signal. If the HO2S fails, this is recognized by the monitor and a fault is stored in the PCM memory.

L3003_02031

| X | Time | Y | Output voltage |
| --- | --- | --- | --- |
| 1 | Amplitude of HO2S signal | 5 | Aged HO2S |
| 2 | Response time of HO2S signal | 6 | Correct signal |
| 3 | Period duration of HO2S signal | 7 | Faulty signal |
| 4 | New HO2S |  |  |

#### HO2S Heater Monitor

- The HO2S heater monitor intermittently checks the heater element of the upstream and downstream HO2S for electrical faults. On vehicles with linear-type HO2S the monitor additionally checks the heater element for functional faults by monitoring the HO2S signal a short time after engine start. If the HO2S heater fails, this is recognized by the monitor and a fault is stored in the PCM memory.

#### TWC Monitor

- The TWC monitor intermittently checks, whether the conversion rate of the TWC is above a certain limit. The conversion rate depends on the ability of the active catalytic coating to store oxygen. Contamination or ageing of the active catalytic coating reduces this ability and hence the conversion rate of the TWC.

- The TWC monitor compares the oxygen concentration in the exhaust gas downstream of the TWC (detected via the downstream HO2S) to that upstream of the TWC (detected via the upstream HO2S) to determine the oxygen storage capacity of the TWC. The PCM counts the number of switches between rich and lean cycles at the downstream and upstream HO2S, and calculates the switch ratio:

Switch ratio = No. of switches at downstream HO2S / No. of switches at upstream HO2S

- A low switch ratio of 0 means that there are no switches at the downstream HO2S, i.e. the oxygen concentration downstream of the TWC is low. As a result, the oxygen storage capacity of the TWC is high and consequently the conversion rate is good.

- A high switch ratio of 1 means that the number of switches at the downstream HO2S is equivalent to the number of switches at the upstream HO2S, i.e. the oxygen concentration downstream of the TWC is equivalent to that upstream of the TWC. As a result, the oxygen storage capacity of the TWC is low and consequently the conversion rate is bad (e.g. active catalytic coating completely ineffective or hole in the ceramic carrier).

- If the TWC fails and subsequently the switch ratio exceeds a certain limit (typically 0.75), this is recognized by the monitor and a fault is stored in the PCM memory.

NOTE:	On vehicles with two catalytic converters arranged in series only the upstream TWC (warm-up TWC) is monitored.

L3003_02032

| 1 | From exhaust manifold | 5 | Upstream HO2S |
| --- | --- | --- | --- |
| 2 | TWC | 6 | Signal of the upstream HO2S |
| 3 | To tailpipe | 7 | Normal signal of the downstream HO2S |
| 4 | Downstream HO2S | 8 | Signal of the downstream HO2S when the TWC is aged |

#### EGR System Monitor

- The Tribute F/L features an EGR system monitor, which intermittently checks the components of the EGR system for electrical faults and functional faults. The monitoring strategy for functional faults varies depending on the engine type.

- On the Tribute F/L with L3 engine (equipped with stepper motor-controlled EGR valve) the EGR system is checked for functional faults by monitoring the pressure changes in the intake manifold (derived from the MAP sensor signal) when opening and closing the EGR valve several times.

L3003_02034

| X | Time | Y1 | EGR valve lift |
| --- | --- | --- | --- |
| Y2 | Manifold absolute pressure |  |  |

- On the Tribute F/L with AJ engine (equipped with vacuum-controlled EGR valve) the EGR system is checked for functional faults by monitoring the EGR rate in the EGR pipe (derived from the EGR differential pressure sensor signal).

- If an EGR system-related component fails, this is recognized by the monitor and a fault is stored in the PCM memory.

NOTE: On all other current Mazda vehicles (equipped with stepper motor-controlled EGR valve) the EGR system is solely monitored for electrical faults via the CCM.

#### Temporary Disablement of Monitors

- According to the EOBD regulations temporary disablement of individual monitors is permitted under certain conditions to prevent mis-diagnosis and hence incorrect malfunction indication by the EOBD system. The monitors can be disabled during:

- Vehicle operation with a fuel tank level below 20 %

- Vehicle operation at elevations over 2500 m above sea level

- Engine start at ambient temperatures below -7 °C

#### Monitor Status

- The monitor status provides information about the monitors supported by the EOBD system and their completion status.

L3003_02024

- The monitors supported by the EOBD system can be checked with the aid of the PIDs with the ending "_SUP". If e.g. the PID EGR_SUP is set to "YES", the EGR system monitor is supported. Monitors that are set to "NO" are generally not supported and can be disregarded.

- The completion status of the monitors can be checked with the aid of the PIDs with the ending "_EVAL". If e.g. the PID EGR_EVAL is set to "YES", the tests of the EGR system monitor have been completed.

NOTE:	For monitors that are not supported by the EOBD system the PIDs with the ending "_EVAL" are automatically set to "YES“. If e.g. the evaporative system monitor is not supported on Mazda petrol engines, the PID EVAP_SUP is set to "NO" and the PID EVAP_EVAL is set to "YES". This is to prevent technicians from mis-interpreting that the tests of this monitor have not been completed yet.

- CCM, misfire monitor and fuel system monitor run continuously and therefore always display "YES“ for the completion status. The intermittently operating HO2S, HO2S heater, TWC and EGR system monitors only display "YES", if the respective tests have been completed at least once.

- For non-continuous monitors the completion status is reset to "NO" when clearing the DTCs with M-MDS. Then the OBD drive mode must be performed, so that these monitors can carry out their tests.

- If "NO" is displayed for a non-continuous monitor, the following reasons should be considered:

- No monitoring has been performed since the vehicle was manufactured.

- No monitoring has been performed since the fault memory was last cleared.

- A fault has been detected during monitoring.

- The monitor status and other EOBD-related PIDs can be checked with M-MDS via the option ToolboxPowertrainOBD Test ModesMode 1 Powertrain Data.

### MIL Activation and Fault Storage

- The conditions for MIL activation and fault storage largely depend on the vehicle type. Generally the MIL only comes on when a fault is stored in the PCM memory as a confirmed fault. If the fault does not reoccur during three drive cycles, the MIL is extinguished in the fourth drive cycle. However, the fault remains stored in the PCM memory. Faults that no longer occur are automatically cleared from the PCM memory after 40 warm-up cycles.

- In addition, the MIL is extinguished when the related fault is cleared with M-MDS.

L3003_02025

- If the MIL illuminates or flashes, the customer is not legally obliged to do anything, but the fault that caused the MIL to illuminate continuously or to flash may result in the vehicle failing a regular emission test. However, the customer is advised to take the vehicle to an authorized Mazda workshop as soon as possible.

- In addition, the EOBD system records the distance traveled since the MIL was activated. Thus a check can be made on how long a customer took to have an emission-relevant fault repaired. This may become important in warranty issues and therefore should be noted on the repair order.

- The distance traveled since activation of the MIL can be checked with the aid of the PID MIL_DIS (Meter), which can be called up with M-MDS via the option Toolbox PowertrainOBD Test ModesMode 1 Powertrain Data.

- On all current Mazda vehicles with Z-type or L-type engines the MIL is activated either after one drive cycle or after two drive cycles. In addition, on the Tribute F/L the MIL is generally activated after two drive cycles.

- MIL activation after one drive cycle is used for faults for which fail-safe data is stored in the PCM. If such a fault comes up, the PCM reverts to the fail-safe data (e.g. substitute value) for engine control. As a result, a fault occurring for the first time is stored in the PCM memory directly as a confirmed fault (MIL DTC). Consequently, the MIL is illuminated.

- MIL activation after two drive cycles is used for faults for which no fail-safe data is stored in the PCM. If such a fault comes up, the PCM initiates a limp home mode (e.g. engine speed limitation). As a result, a fault occurring for the first time is stored in the PCM memory as a presumed fault (pending DTC), but the MIL is not illuminated. If the fault is not confirmed during the second drive cycle, the PCM judges that the system/ component has returned to normal operation or that the fault was detected due to mis-diagnosis, and deletes the presumed fault.

- Presumed faults related to minor misfire or the fuel system are only deleted, if they are not confirmed during a second drive cycle that meets all of the following conditions:

- Engine speed within ± 375 min-1 of that during the first drive cycle

- Engine load within ± 20 % of that during the first drive cycle

- Engine warm-up status same as that during the first drive cycle (i.e. engine coolant temperature either below 70 °C (cold engine) or 70 °C or above (warm engine))

- However, if such a fault is confirmed during the second drive cycle, the PCM judges that the system/component has failed, and stores a confirmed fault in addition to the presumed fault. As a result, the MIL is illuminated.

- On the Mazda2 with MZI engine the MIL is generally activated after three drive cycles. As a result, a fault occurring for the first time is stored in the PCM memory as a presumed fault, but the MIL is not illuminated. If the fault is not confirmed during the second drive cycle, the PCM judges that the system/component has returned to normal operation or that the fault was detected due to mis-diagnosis, and deletes the presumed fault (also refer to MIL activation after two drive cycles).

- However, if such a fault is confirmed during the second and the third drive cycle, the PCM judges that the system/component has failed, and stores a confirmed fault in addition to the presumed fault in the third drive cycle. As a result, the MIL is illuminated.

- Independent from the vehicle type the MIL is activated immediately during the first drive cycle, if severe misfire causing irreversible damage to the TWC is detected. In this case, a confirmed fault is stored in the PCM memory and the MIL flashes.

- The pending DTCs and the MIL DTCs can be read out and cleared with M-MDS via the option ToolboxSelftestModulesPCMRetrieve CMDTCs.

NOTE:	When the DTCs are cleared, the freeze frame data, diagnostic monitoring test results and distance traveled since activation of the MIL are cleared as well. In addition, the completion status of the non-continuous monitors is reset.

NOTE:	On the Mazda2 and Tribute F/L the DTC P1000 (OBD system readiness test not complete) comes up after the PCM memory has been cleared. This fault code indicates that not all monitors have completed their tests. The fault code does not mean that a fault has been found in a system/component, and can therefore not be cleared with M-MDS. Once the OBD drive mode has been performed and all monitors have completed their tests, the DTC P1000 is cleared automatically.

### Freeze Frame Data

- The FFD (Freeze Frame Data) is a snapshot of the engine operating conditions at the occurrence of the first fault. This data is stored in the PCM memory and will not be overwritten, even if a confirmed fault is stored in addition to a presumed fault. The only exception are faults related to misfire or the fuel system, since these have a higher priority. If such a fault is stored in the PCM memory, its FFD will overwrite any old data, unless the previous FFD are also related to misfire or the fuel system.

L3003_02026

- The FFD is very helpful when diagnosing the potential causes of a malfunction. In addition, a concern can be reproduced easier by driving the vehicle under the same conditions as indicated by the FFD.

- According to the EOBD regulations the following data must be stored in the PCM memory when the first fault is detected:

- The FFD can be read out with M-MDS via the option ToolboxSelftestModules PCMRetrieve CMDTCs.

NOTE:	Depending on the vehicle type FFD for a pending DTC is only displayed by M-MDS, if the fault in question is also stored as a confirmed fault in the PCM memory.

### Diagnostic Monitoring Test Results

- The DMTR (Diagnostic Monitoring Test Results) are the numerical results from the last tests performed by certain monitors. This data is stored in the PCM memory and will only be overwritten whenever a monitor has completed its tests again. The data consists of a description of the corresponding test, the OBDMID (OBD Monitor Identification), test ID, minimum/maximum limit and test result.

L3003_02027

- The DMTR provide additional information, whether monitoring of a component/system has been completed and whether the component/system is okay. Therefore, the test results of the monitor in question must be compared to the respective minimum/ maximum limit:

- If the DMTR and the minimum/maximum limit indicate the initial values (e.g. 0), monitoring has not been performed at all or has not been completed yet. The completion status of the corresponding monitor is consequently set to “NO”. In this case, the OBD drive mode required for the monitor in question must be repeated.

- If the DMTR indicate a value which is outside the limits, monitoring is completed and the component/system in question is faulty. The completion status of the corresponding monitor is consequently set to “NO”, and a pending DTC or MIL DTC is stored in the PCM memory.

- If the DMTR indicate a value which is within the limits, monitoring is completed and the component/system in question is okay. The completion status of the corresponding monitor is consequently set to “YES”.

- The DMTR can be read out with M-MDS via the option ToolboxPowertrain 
OBD Test ModesMode 6 On-board Test Results.

NOTE:	The DMTR displayed by M-MDS are not updated even when the data stored in the PCM memory changes. For this reason, the Repeat button must be pressed to update the M-MDS display.

### Tamper Protection

- According to the EOBD regulations the PCM/TCM must have a write-protection for any reprogrammable computer code to prevent tampering (such as unauthorized reprogramming etc.).

- The CALID (CALibration Identification) identifies the specific calibration and is defined as a 16-digit alphanumerical code providing information about calibration name, type of vehicle, release date, release engineer and version number. The CVN (Calibration Verification Number) serves to verify whether the correct software is stored in the PCM/TCM and is similar to an encrypted checksum. CALID and CVN will be tracked for all initial releases, running changes and field fixes of the PCM/TCM software.

L3003_02028

- The CALID, CVN and VIN can be read out with M-MDS via the option Toolbox PowertrainOBD Test ModesMode 9 Vehicle Information.

### Diagnostics

- When performing diagnosis and repair of EOBD-related concerns the basic procedure shown below should be followed.

NOTE:	Failure to follow this procedure will make it extremely difficult to properly diagnose and repair EOBD-related concerns.

#### OBD Drive Mode

- The OBD drive mode consists of the PCM adaptive memory learning mode and the repair verification drive mode, and serves to verify that the vehicle is in order after performing any EOBD-related repairs.

#### PCM Adaptive Memory Learning Mode

- The PCM adaptive memory learning mode allows the engine management system to establish the permanent correction factors of the long-term fuel trim. The PCM uses these values for the air/fuel ratio control and for the OBD monitors.

- If the PCM adaptive memory learning mode is not performed, most monitors will never run. For this reason, it is absolutely necessary to conduct the learning mode prior to the repair verification drive mode.

- On all current Mazda vehicles except for the Mazda2 the permanent correction factors for the long-term fuel trim are logged in the KAM. Since the Mazda2 features no KAM, the correction factors are logged in the FEEPROM.

NOTE:	After performing EOBD-related repairs the permanent correction factors of the long-term fuel trim should be reset by disconnecting the battery for 1 min. Hereafter, the PCM adaptive memory learning mode must be performed, allowing the PCM to 
re-establish the correction factors.

NOTE:	On the Mazda2 and Tribute F/L the PCM adaptive memory learning mode is not available.

- E.g. the PCM adaptive memory learning mode for Mazda3 BK and Mazda6 GG/GY with 2.0 MZR engine must be performed as following:

- Bring the engine to normal operating temperature and verify that all accessory loads (A/C, headlights, blower motor, rear window defroster) are off.

- Make sure that ignition timing and idle speed are within specification.

- Increase the engine speed according to the learning mode indicated in the figure. Hereafter, let the engine idle for more than 30 s after the cooling fan has stopped.

- Confirm that the PCM adaptive memory learning mode has been performed successfully by checking the PID RFC_FLAG (Mode), which can be called up with 
M-MDS via the option ToolboxDataloggerModulesPCM. If the PID is set to “Learnt”, the learning mode has been performed successfully. Otherwise repeat the PCM adaptive memory learning mode.

NOTE:	Depending on the vehicle type the PID RFC_FLAG may not be available.

L3003_02017

| X | Time | Y | Engine speed |
| --- | --- | --- | --- |
| 1 | Idle | 3 | Idle with cooling fan OFF |
| 2 | 2800…3200 min-1 |  |  |

#### Repair Verification Drive Mode

- The repair verification drive mode is designed in a way that all the monitors can conduct and complete their tests in a logical order and hence in the fastest possible way.

NOTE:	The tests of the monitors can be completed faster if the accelerator pedal is moved smoothly when accelerating or decelerating. In addition, the fuel tank level should be above 20 % as otherwise the misfire monitor, fuel system monitor, HO2S monitor and TWC monitor might be disabled, preventing other monitors from running.

- E.g. the repair verification drive mode for the Mazda3 BK with 2.0 MZR engine and MTX must be performed as following:

- Confirm that the PID RFC_FLAG is set to “Learnt”. If the PID is set to “Not Learnt”, perform the PCM adaptive memory learning mode prior to the repair verification drive mode.

- Bring the engine to normal operating temperature and verify that all accessory loads (A/C, headlights, blower motor, rear window defroster) are off.

- Drive the vehicle according to the drive mode indicated in the figure. First drive in zone O, then A or B, followed by C or D and finally E or F. The driving conditions before driving at constant vehicle speed are not specified.

- Confirm that the repair verification drive mode has been performed successfully by checking the completion status of the monitors. If all PIDs with the ending “_EVAL” are set to “YES”, the drive mode has been performed successfully. Otherwise repeat the repair verification drive mode.

L3003_02018

| X | Time | Y | Vehicle speed |
| --- | --- | --- | --- |

- On the Tribute F/L the instructions for the repair verification drive mode can be found in M-MDS. In addition, the M-MDS allows the user to check the current operating conditions and the status of the individual monitors.

L3003_02029

- The instructions for the repair verification drive mode can be accessed with M-MDS via the option ToolboxPowertrainOBD Test ModesOBD Drive Cycle.

## Notes:

## Engine Mechanical System

### General

- While the various sub-systems of the Mazda petrol engine management systems (such as intake-air system, fuel system, ignition system, emission system and control system) have already been covered in the course “Basic Petrol Engine Management” and in the preceding sections, this section describes the engine mechanical system.

- The following parameters of the engine mechanical system can affect the running of a petrol engine:

- Compression pressure

- Valve timing

- Valve clearance

### Compression Pressure

- A sufficiently high compression pressure ensures that the air/fuel mixture burns completely in the combustion chamber.

- Insufficient compression pressure can occur due to worn piston rings, worn valves or a leaking cylinder head gasket. This can lead to poor engine power and increased exhaust emissions.

L3003_02019

| 1 | Compression pressure tester |  |  |
| --- | --- | --- | --- |

### Valve Timing

- The correct valve timing ensures that the valves open and close at the right time to provide a good cylinder charge and optimum compression pressure.

- Incorrect valve timing can occur, if the camshaft sprockets are not aligned correctly or if the timing belt has jumped over. On engines with variable valve timing a faulty camshaft actuator can also cause incorrect valve timing. This can lead to misfire, poor engine power, increased exhaust emissions, or engine damage due to contact between the piston and the valves (the latter one depending on the engine type).

L3003_02020

| 1 | Timing mark for exhaust camshaft | 3 | Timing mark for crankshaft |
| --- | --- | --- | --- |
| 2 | Timing mark for intake camshaft |  |  |

### Valve Clearance

- The correct valve clearance ensures that the valves open wide enough to provide a good cylinder charge and at the same time close fully to ensure an optimum compression pressure.

- Incorrect valve clearance can occur due to wear and thermal overloading of the valves. On engines with hydraulic valve clearance compensation, faulty hydraulic lash adjusters can also cause incorrect valve clearance. This can lead to poor engine power, noises from the valve gear, or burnt valves due to insufficient heat transfer to the cylinder head.

L3003_02021

### Diagnostics

- The engine mechanical system can be checked as following:

- Checking the compression pressure

- Checking the pressure loss

- Checking the valve timing

- Checking the valve clearance

#### Checking the pressure loss

- If the compression pressure of a cylinder is found to be too low, the pressure loss test can be used to locate the leak in the combustion chamber and to establish which components are faulty. The principle of the pressure loss test is that the combustion chamber is pressurized with compressed air and the loss of pressure arising due to the leak is indicated by flowing noises. For this, the piston must be in TDC position and the valves closed. Then a compressed air hose is connected with a suitable adapter to the spark plug opening of the cylinder concerned.

- If the compressed air is escaping at a valve, flowing noises will be audible in the intake-air or exhaust system. Loss of pressure past the piston rings into the crankcase can be identified by flowing noises from the oil filler opening. Flowing noises in another cylinder or bubbling in the cooling system indicates a leaking cylinder head gasket.

## Diagnostic Process

### General

- Diagnosis requires comprehensive knowledge of the system operation. As with all diagnosis, a technician must use symptoms and clues to determine the cause of a customer concern. The following diagnostic process provides you with a logical method for rectifying customer concerns:

- Confirm the symptom of the customer concern.

- Determine which system of the vehicle could be causing the symptom.

- Once you identify the particular system, determine which component(s) within that system could be the cause for the symptom.

- After determining the faulty component(s) you should always try to identify the cause of the failure. In some cases components just wear out. However, in other cases something else than the failed component is responsible for the problem.

L3003_02022

| 1 | Symptom | 3 | Components |
| --- | --- | --- | --- |
| 2 | Systems | 4 | Causes |

- For example, a customer’s car is brought in the workshop with a “No start” concern, i.e. the symptom is that the engine will not start. During diagnosis you find that the fuel pressure in the fuel rail is too low. Therefore, you determine that the fuel system is the cause for the problem. By performing diagnostic routines, you determine that the fuel pump is the faulty component. Further investigation shows that contamination in the fuel tank is the cause of the component failure.

### Basic Checks for Troubleshooting

- When performing troubleshooting on a petrol engine the following basic checks should be made before moving on to more complex electrical checks.

NOTE:	If the customer concern is “No start” or “Hard start”, spray “Start Pilot” into the intake-air duct while cranking the engine. If the engine starts, the fuel system might be faulty. If the engine does not start, then this indicates a fault in the ignition system or engine mechanical system.

#### Engine Mechanical System

- Check the compression and if necessary the pressure loss.

- Check the valve timing.

- Check the valve clearance.

- Check the engine oil for contamination (e.g. by engine coolant).

- Check the engine coolant for contamination (e.g. by engine oil).

#### Intake-air System

- Check the condition of the air cleaner.

- Check the intake-air system for leakage (incl. O-rings of the injectors) or oil ingress.

- Check the adjustment of the accelerator cable (only mechanical throttle valve).

- Check the function of the electronic throttle valve.

- Check the function of the idle air control valve (only mechanical throttle valve).

- Check the idle speed.

- Check the turbocharger (only 2.3 MZR DISI Turbo engine).

- Check the function of the boost pressure control valve (only 2.3 MZR DISI Turbo engine).

#### Fuel System

- Check whether the fuel tank contains sufficient petrol.

- Check the petrol for contamination (e.g. by particles, water, diesel fuel).

- Check the fuel system for leakage.

- Check the fuel lines for leakage or kinks.

- Check the function of the fuel tank ventilation.

- Check the function of the fuel pump.

- Check the fuel pressure.

#### Ignition System

- Check the condition of the spark plugs.

- Check condition and resistance of the high-tension leads.

- Check the ignition voltage.

- Check the ignition timing.

#### Emission System

- Check the exhaust system for leakage.

- Check the catalytic converter for blockage.

- Check the function of the EGR system.

- Check the function of the evaporative emissions control system.

- Check the function of the positive crankcase ventilation system.

#### Control System

- Check the function of the fuses and relays.

- Check the condition of the electrical connections incl. ground connections.

- Check the condition of the battery.

- Check the condition of the starter motor.

- Check the function of the variable valve timing system.

## Notes: