## Introduction

- Ever stricter exhaust and noise emission regulations, and the demand for high power output and low fuel consumption impose increasing demands on the engine management system of the petrol engine. In order to fulfill these requirements the combustion process is improved continuously.

- This course is a theoretical and practical guide to gain general and Mazda specific knowledge about direct injection systems, including their components, operation and diagnosis. In addition, the course provides comprehensive information about the 
on-board diagnostic system (incl. EOBD), the engine mechanical system and the diagnostic process.

- While the manifold injection systems used on Mazda vehicles have already been covered by the course “Basic Petrol Engine Management” (CT-L2004), this course describes the more complex direct injection system employed on the Mazda3 MPS and Mazda6 MPS with 2.3 MZR DISI Turbo engine.

- Anyone associated with the diagnosis and repair of petrol engine management systems must have the knowledge to deliver a “Fix it right first time” repair. Therefore, the Mazda Masters development and qualification path provides the following training courses required for diagnosing and repairing petrol engine management systems:

- Basic Petrol Engine Management CT-L2004

- Advanced Petrol Engine Management CT-L3003

- The ranking of this course within the Mazda Masters educational system is Level 3 - ‘Mazda Master Technician’. It is intended for technicians who have experience in maintaining and repairing Mazda vehicles, and who are familiar with the features and the operation of the various manifold injection systems.

## Fundamentals

### Turbocharged Engine with Direct Injection

- In comparison to a naturally aspirated engine with the same power output the 2.3 MZR DISI (Direct Injection Spark Ignition) Turbo engine has a considerably smaller displacement, achieving the following advantages:

- Reduced pumping loss due to less volume swept during each working cycle

- Reduced heat transfer to the cylinder wall due to its smaller surface area

- Reduced friction loss due to the smaller size of the moving parts (piston, piston rings, connecting rod etc.)

- These measures result in lower fuel consumption, especially at part load. In addition, the engine is more compact and lightweight, improving handling of the vehicle.

- In order to increase the engine torque and hence the power output across the entire engine speed range, the charging efficiency is increased with the aid of a turbocharger. However, turbochargers have serious disadvantages in terms of exhaust emissions and fuel consumption.

- Basically, in a turbocharged engine it takes longer until the catalytic converter reaches its operating temperature, i.e. the exhaust emissions immediately after engine start are higher than on a naturally aspirated engine. This is due to the fact that the turbocharger absorbs large amounts of exhaust heat, reducing the exhaust gas temperature.

- At the same time, there is an adverse effect on the thermal efficiency due to the low compression ratio of the turbocharged engine, i.e. the fuel consumption is higher compared with a naturally aspirated engine. However, it is impracticable to set a high compression ratio, since the higher temperature in the combustion chamber of the turbocharged engine can easily lead to engine knocking.

- The adoption of direct injection is the solution to these problems of the turbocharged engine. Through the injection of a finely atomized fuel spray directly into the combustion chamber a homogeneous air/fuel mixture forms in the vicinity of the spark plug, ensuring stable combustion. This allows the ignition timing to be retarded, enabling a high exhaust gas temperature even with a turbocharger installed. As a result, the catalytic converter reaches its operating temperature faster, achieving low exhaust emissions immediately after engine start.

- Furthermore, direct injection improves the internal cooling effect in the combustion chamber, since the fuel evaporates solely in the cylinder (and not already in the intake port as on a turbocharged engine with manifold injection). In this way, engine knocking is prevented even with a turbocharger installed. As a result, the compression ratio can be set as high as for a naturally aspirated engine, reducing the fuel consumption.

L3003_00001

| 1 | 2.3 MZR engine | 3 | Injector |
| --- | --- | --- | --- |
| 2 | 2.3 MZR DISI Turbo engine | 4 | Spark plug |

## Notes: